LEVEL D TEST 1 CODE 11

| Which traffic sign shows the maximum permitted bogie load?. A. B. C. D. | 10652 |
| --- | --- |
| Which traffic signs are you allowed to pass when driving a light goods vehicle with a coupled trailer? The trailer has an unarticulated towbar.. Traffic signs A and C. Traffic signs A and B. Traffic signs B and C. Traffic signs C and D. Traffic signs B and D. | 11796 |
| You are driving at 70 km/h. How far does your vehicle travel in one second?. Approx. 10 metres.. Approx. 20 metres.. Approx. 25 metres.. Approx. 35 metres.. |

| You are driving at 70 km/h. How far does your vehicle travel in one second?. Approx. 10 metres.. Approx. 20 metres.. Approx. 25 metres.. Approx. 35 metres.. |
| --- |
| You are going to help to start another car by using jump leads. Which statement is correct?. If the jump leads are connected incorrectly, there may be a short circuit.. The batteries should be of different voltages. The engine of the car providing starting help should be turned off. The car which is having problems starting should have a gear engaged. | 10578 |  |  |  |  |  |  |  |  |  |
| Which traffic sign indicates a limited axle load?. Traffic sign A. Traffic sign B. | 8617 |  |  |  |  |  |  |  |  |  |
| Why should the tyres with the best grip be fitted at the rear?. They are under most strain because of the weight of load in the baggage compartment. They are under most strain if I brake hard. The car's road-holding characteristics will be better. It shortens the braking distance. |
| What does this combination of traffic signs mean?. Limited width of vehicle. Minimum permitted distance to the vehicle in front. Limited width of lane. Vehicles with a width of less than 2.6 meter must have an exemption to drive on the road.. | 2827 |  |  |  |  |  |  |  |  |  |
| Which lamp lights up if the airbag system is not working?. Lamp no. 1. Lamp no. 2. Lamp no. 9. Lamp no. 10. | 1289 |  |  |  |  |  |  |  |  |  |
| You are going to jump-start your car by using electricity from another car's battery. Which of the following is correct?. The minus terminals must be linked to the plus terminals. The batteries that are linked up must have the same voltage. The car that I am taking electricity from must not have its engine on. |
| When driving, you notice that the temperature gauge has suddenly gone up to the red area. What is the first thing you should do?. Stop and check that there is enough coolant in the cooling system. Stop and check the oil level in the engine. Drive to the nearest garage. | 10571 |  |  |  |  |  |  |  |  |  |
| Which equipment must a car have according to the regulations?. Jack.. Tachometer.. Two red reflectors at the rear.. |
| Which statement is correct?. The use of seat belts increases the risk of injury or death if I turn my car over. The use of seat belts implies risks in the case of a side collision. The use of seat belts reduces the risk of injury or death in traffic by 40-60 %. The use of seat belts reduces the risks if I have the three-point belt lightly stretched high up on my neck. |
| Are you allowed to drive a car whose foot brake is not working?. Yes, the shortest possible route to a garage. Yes, if you use the hazard warning lights. No, I must call in a breakdown truck. |
| In certain batteries you should check the liquid level. Which liquid should you top up the battery with when necessary?. Sulphuric acid.. Distilled water.. Methylated spirit.. Salt-solution.. |
| Which traffic sign means that you must be well aware of the height of the vehicle you are driving?. Traffic sign A.. Traffic sign B.. Traffic sign C.. Traffic sign D.. | 2756 |  |  |  |  |  |  |  |  |  |
| When is there the greatest risk of carbon monoxide poisoning?. If I drive with the wrong fuel.. If I run the car warm for more than one minute.. If I reverse into a snowdrift so that the exhaust pipe becomes blocked.. If I have the side window open.. |
| One of the car's brake circuits is defective. Which lamp warns you of this?. Lamp no. 3. Lamp no. 6. Lamp no. 9. Lamp no. 10. | 1286 |  |  |  |  |  |  |  |  |
| Which traffic sign or signs provide information on the maximum gross weight of a vehicle or vehicle train that is allowed to pass?. Only sign A. Only sign B. Traffic sign A and C. Traffic sign B and D. Traffic sign A and B. | 8859 |  |  |  |  |  |  |  |  |
| You double your speed. How much greater will the kinetic energy be?. Twice as much.. Three times as much.. Four times as much.. Five times as much.. |
| You can feel vibrations in the steering wheel when you drive at a certain speed. What is the most common reason for this problem?. The front end needs lubricating. Tyre pressure in the front tyres too high. Unbalanced front wheels. | 10506 |  |  |  |  |  |  |  |
| You increase speed from 30 km/h to 90 km/h. How does this affect the braking distance?. It remains unchanged. It becomes three times as long. It becomes six times as long. It becomes nine times as long. |
| Where is there information on which type of oil should be used for the engine?. The instruction manual.. The registration certificate.. The motor vehicle inspection report.. |
|  |  |  |  |  |  | The thinking distance increases in relation to the speed. What is correct? |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Twice the speed gives a thinking distance twice as long. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Twice the speed gives a thinking distance three times as long. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Twice the speed gives a thinking distance four times as long. |  |  |  |  |  |  |  |  |  |  |  |  |
| How far does the car travel in one second?. At 30 km/h the car travels approx. 8 metres.. At 50 km/h the car travels approx. 22 metres.. At 70 km/h the car travels approx. 32 metres.. At 80 km/h the car travels approx. 34 metres.. |
| There is not enough fluid in the car battery. What should you top up with?. Distilled water. Carburettor spirit. Antifreeze. | 10509 |  |  |  |  |  |  |
| Which of the following statements about a car's warning lights is true?. If the charging system lamp comes on while I am driving, I must stop immediately and check the fluid level of the battery. If the temperature gauge suddenly rises to boiling point, I must stop immediately and check the engine's oil level. If the red oil pressure lamp comes on while I am driving, I must stop immediately and turn off the engine. | 11925 |  |  |  |  |
| You are driving a vehicle whose height exceeds 2.5 m. What does this traffic sign combination mean?. Driving straight ahead is permitted but after 900 m there will be limited available height. I must turn right. Driving straight ahead is prohibited. | 11991 |  |  |  |  |
|  |  |  |  |  |  |  | What is true concerning the engine heater? |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It increases the fuel consumption since a hot engine uses more fuel than a cold engine |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It makes the work of the catalytic converter more difficult |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It reduces the fuel consumption and therefore the emission of exhaust fumes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It should only be used when the temperature outside is below minus |  |  |  |  |  |  |  |  |  |  |  |
| Which vehicle or combination of vehicles may pass this sign?. A car with a trailer. A heavy goods vehicle without a trailer. A heavy goods vehicle with a coupled trailer(semitrailer). | 11804 |  |  |  |
| You skid as a result of too powerful an acceleration in a curve. What is true about the handling?. Front-wheel driven cars skid with the rear of the car. Front-wheel driven cars become oversteered. Rear-wheel driven cars are experienced as understeered. Front-wheel driven cars are experienced as understeered. |
| Which tyre indicates faulty front end wheel alignment?. Tyre A. Tyre B. Tyre C. Tyre D. | 49 |  |
| What does the concept of stopping distance mean?. The distance the car travels during braking.. The total length of the thinking distance and braking distance.. The distance the car travels during my reaction time.. |

| What is true concerning seat belts in cars?. The seat belt must be used even when reversing. A small child sitting on the knee of an adult can be secured in the same seat belt as the adult. The driver is responsible to ensure that children under 15 years of age use seat belts. |
| --- |
| Which statement is true regarding car batteries?. They contain a corrosive acid. They must contain glycol in the winter so as not to freeze.. They work at full capacity even at very low temperatures. | 10576 |  |  |
| Which lamp lights up when the battery is not charging?. Lamp no. 6. Lamp no. 7. Lamp no. 9. Lamp no. 10. | 1292 |  |  |
| Which sign means that you must be well aware of how wide the vehicle you are driving is?. Traffic sign A.. Traffic sign B.. Traffic sign C.. Traffic sign D.. | 2755 |  |  |
| Which of the following statements about seat belts is true?. A seat belt combined with a thick jacket reduces the strain on the body in the event of a collision. A seat belt should lie close to the body and be as tight as possible. |
| How should a child aged two years most safety be placed in a car?. The child should sit on a booster cushion and use the seat belt. The child should sit in a child safety seat mounted either in the back seat or front seat according to the manufacturer's instructions. The child should sit in a protective cot with seat belt. | 229 |  |
| Which traffic sign is applicable to the next crossing?. A. B. C. | 1081 |  |
| What does this traffic sign mean?. Goods vehicles whose maximum load exceeds 3.5 tons are prohibited. Goods vehicles whose gross weight exceeds 3.5 tons are prohibited. Goods vehicles whose kerb weight exceeds 3.5 tons are prohibited. Goods vehicles whose total weight exceeds 3.5 tons are prohibited. | 11805 |  |
| Which statement is correct concerning the driver's responsibility for the use of seat belts?. The driver is always obliged to use the seat belt even in a parking area. The driver of a light lorry does not need to use the seat belt. The driver is responsible to ensure that everyone in the car uses the seat belts. The driver is only responsible to ensure that children less than 15 years of age use a seat belt or special safety device. |
| What is meant by stopping distance?. The distance the car travels from when I perceive the danger until I stop the car.. The distance the car travels from when I start braking until I stop the car.. The distance the car travels from when I perceive a danger to when I start braking.. |
| Lamp no. 6 lights up when you are driving. What should you do?. Press down the clutch and rev up the engine a few times. Continue driving a few kilometres to see if the warning lamp goes off. Stop the engine immediately. | 10510 |

| Which statement is correct?. An airbag is always turned on and off with a switch on the dashboard. If the vehicle has an airbag, you do not need to use a seatbelt. A child car seat must not be used in the front passenger seat if there is an airbag in operation. |
| --- |
| You are driving a car equipped with airbags for both front seats. Which alternative is correct?. It is dangerous to wear glasses if sitting in a seat with an airbag. An airbag is triggered in all collisions. An airbag provides best protection if I do not use the seatbelt. A child safety seat must not be mounted in the front seat. |
| Which lamp lights up when it is time to refuel?. Lamp no. 4. Lamp no. 5. Lamp no. 6. Lamp no. 8. | 1288 |  |  |  |
| Your car's foot brake (service brake) is not working. Are you allowed to drive if the handbrake is working?. No, I must tow the car. Yes, both only at walking speed. Yes, but only to the nearest garage. No, I must call for a breakdown truck. |
|  |  |  |  | What is true about the steering system? |  |  |  |  |  |
|  |  |  |  | If I drive against the edge of the kerb the wheel angles of the front wheels can become changed and the tyres will wear unevenly |  |  |  |  |  |
|  |  |  |  | A car with power steering cannot be steered if the motor is switched off |  |  |  |  |  |
|  |  |  |  | All cars have power steering |  |  |  |  |  |
| There are now wintry conditions on the roads. When must you have winter tyres?. 1 October - 1 April. 1 October - 30 April. 1 December - 1 March. 1 December - 31 March. Always. | 10586 |  |  |
| Which lamp lights up when the engine temperature is too high?. Lamp no. 3. Lamp no. 6. Lamp no. 7. Lamp no. 8. | 1287 |  |  |
| The left front tyre on my car has been excessively worn on one side of the tread. What is true?. The air pressure in the tyre has been too low. The air pressure in the tyre has been too high. The car's wheel alignment is defective. |

|  |  |  | How much longer will the braking distance be if, in specific road conditions, you increase the speed from 35 km/h to 70 km/h? |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Twice as long. |  |  |
|  |  |  | Three times as long. |  |  |
|  |  |  | Four times as long. |  |  |
|  |  |  | Nine times as long. |  |  |
| You are driving at 70 km/h. How far does your vehicle travel in one second?. Approx. 10 m. Approx. 20 m. Approx. 30 m. Approx. 40 m. |
| What influences the reaction time?. The condition of the driver. The condition of the vehicle. The road conditions. The speed. |
| Your car has no child safety-seat. Is there ever an occasion when you may transport a child who is 4 years old and about 100 cm long?. Yes, no matter the distance, as a one-off I may transport a child in the backseat. Yes, but only a short distance and only if child sits in the back seat. No, children must always travel in a special child safety-seat. |

| You are going to connect the jump leads from an auxiliary battery to your own car to get it started. What is the correct thing to do first?. Connect the auxiliary battery's positive terminal to your battery's minus terminal.. Connect the auxiliary battery's positive terminal to your battery's positive terminal.. Connect the auxiliary battery's plus terminal to part of the bodywork on your own vehicle.. Connect the auxiliary battery's minus terminal to your own battery's plus terminal.. |
| --- |
| During what period are you normally allowed to use studded tyres?. 1 October - 31 March. 1 October - 15 April. 1 November - 1 April. 1 November - first Sunday after Easter. | 12773 |
| Which fluid do you mix with water in order to prevent ice from forming in the cooling system?. A. B. C. D. | 10593 |

| When must the height of a viaduct be marked out?. Always. When the height available is higher than 4.5 metres. When the height available is lower than 4.5 metres. | 12324 |  |
| --- | --- | --- |
| What is true?. If there is too little air in the left-hand front tyre the car pulls to the right. If there is too little air in the right-hand front tyre the car pulls to the right. |
| Which statement about baby seats in cars is true?. It must be fitted so it rests against the vehicle's dashboard. It must always be fitted in the rear seat. It is the seat belt that restrains the baby seat in a collision. | 12271 |

| Which vehicles must be equipped with these reflector signs?. Heavy goods vehicles with a total weight exceeding 3.5 tons. All heavy goods vehicles. All trailers. Trailers with a total weight exceeding 3.5 tons. | 11674 |  |
| --- | --- | --- |
| What is true concerning seat belts in a car?. The seat belt must be used when reversing. The seat belt must be used when driving in a parking area. Taxi drivers do not need to wear seat belts. The driver is obliged to ensure that passengers less than 15 years old use seat belts or special safety devices for children. |
| Which of these accessories should there be in a car?. Spare tyre. Jump leads. Triangular warning sign. Towline. |
|  |  |  | You increase speed from 30 km/h to 60 km/h. How does this affect the kinetic energy? |  |  |  |
|  |  |  | It remains unchanged |  |  |  |
|  |  |  | It becomes twice as large |  |  |  |
|  |  |  | It becomes three times as large |  |  |  |
|  |  |  | It becomes four times as large |  |  |  |

| Which warning lamp goes on if the oil pressure is too low?. Lamp no. 3. Lamp no. 6. Lamp no. 8. Lamp no. 9. | 10511 |  |
| --- | --- | --- |
| Which combination is correct?. Glycol - the radiator.. Carburettor spirit - the battery.. Distilled water - the radiator.. |
| What is the minimum height a person must be in order to sit in a front passenger seat fitted with an airbag?. At least 100 cm. At least 120 cm. At least 140 cm. At least 160 cm. |
| What is meant by stopping distance?. It is the distance my vehicle travels from when I see danger to when the vehicle stands still. It is the distance my vehicle travels from when I start braking until the vehicle stands still. It is the distance my vehicle travels from when I see danger to when I start braking. |
| What is the advantage of driving a car with ABS brakes?. The brakes cannot freeze. There is less risk of skidding when braking in an emergency. The braking distance is much shorter when braking hard on any road surface. | 10582 |

LEVEL C TEST 2 CODE 12

| Which traffic sign means that you must be well aware of the height of the vehicle you are driving?. Traffic sign A.. Traffic sign B.. Traffic sign C.. Traffic sign D.. | 2756 |  |
| --- | --- | --- |
|  |  |  | What is correct about children in cars? |  |  |  |
|  |  |  | Children can withstand higher collision forces than adults |  |  |  |
|  |  |  | The fitting of a child safety seat must be carried out by an authorised workshop |  |  |  |
|  |  |  | Children less than 10 years old are not included in the seat belt act |  |  |  |
|  |  |  | A child of 20 kg can in the moment of collision "weigh" approx. 1,000 kg if the speed of the vehicle is 50 km/h |  |  |  |
| What is meant by stopping distance?. The distance the car travels from when I perceive the danger until I stop the car.. The distance the car travels from when I start braking until I stop the car.. The distance the car travels from when I perceive a danger to when I start braking.. |
| The thinking distance increases in relation to the speed. What is correct?. Twice the speed gives a thinking distance twice as long.. Twice the speed gives a thinking distance three times as long.. Twice the speed gives a thinking distance four times as long.. |

|  |  |  |  | Which of these accessories should there be in a car? |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Spare tyre |  |  |  |  |
|  |  |  |  | Jump leads |  |  |  |  |
|  |  |  |  | Triangular warning sign |  |  |  |  |
|  |  |  |  | Towline |  |  |  |  |
| Which fluid is used to top up the battery, if necessary?. A. B. C. D. | 10577 |  |
| Which lamp lights up when the battery is not charging?. Lamp no. 6. Lamp no. 7. Lamp no. 9. Lamp no. 10. | 1292 |  |
|  |  |  |  |  | You notice that your car is pulling to one side. What could be the reason? |  |  |  |
|  |  |  |  |  | The power steering is not working |  |  |  |
|  |  |  |  |  | High air pressure in both front tyres |  |  |  |
|  |  |  |  |  | Low air pressure in a front tyre |  |  |  |

| Which of the following must a car be fitted with?. Spare wheel. Pre-engine heater. Cruise control. Catalytic converter. | 12128 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| What is true concerning the engine heater?. It increases the fuel consumption since a hot engine uses more fuel than a cold engine. It makes the work of the catalytic converter more difficult. It reduces the fuel consumption and therefore the emission of exhaust fumes. It should only be used when the temperature outside is below minus. |
| Do you have to wear a seat belt when travelling in a taxi?. No. Yes. | 1365 |  |  |  |  |  |
| Which of the following statements about a car's warning lights is true?. If the charging system lamp comes on while I am driving, I must stop immediately and check the fluid level of the battery. If the temperature gauge suddenly rises to boiling point, I must stop immediately and check the engine's oil level. If the red oil pressure lamp comes on while I am driving, I must stop immediately and turn off the engine. | 11925 |  |  |  |  |  |
| Which traffic sign or signs provide information on the maximum gross weight of a vehicle or vehicle train that is allowed to pass?. Only sign A. Only sign B. Traffic sign A and C. Traffic sign B and D. Traffic sign A and B. | 8859 |  |  |  |  |  |
| Which traffic sign is applicable to the next crossing?. A. B. C. | 1081 |  |  |  |  |  |
| When you are driving in curves it is important to keep to a sufficiently low speed. Which statement is true?. Twice the speed gives twice the lateral force. Twice the speed gives four times the lateral force. Twice the speed gives eight times the lateral force. |
|  |  |  |  |  |  | Which statement is correct concerning the driver's responsibility for the use of seat belts? |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The driver is always obliged to use the seat belt even in a parking area |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The driver of a light lorry does not need to use the seat belt |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The driver is responsible to ensure that everyone in the car uses the seat belts |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The driver is only responsible to ensure that children less than 15 years of age use a seat belt or special safety device |  |  |  |  |  |  |  |  |  |
| How can you detect that the ABS system is not functioning?. The braking distance becomes twice as long as normal. A warning lamp on the dashboard lights up. Harder pressure is needed on the brake pedal. |
| What does this traffic sign mean?. Goods vehicles whose maximum load exceeds 3.5 tons are prohibited. Goods vehicles whose gross weight exceeds 3.5 tons are prohibited. Goods vehicles whose kerb weight exceeds 3.5 tons are prohibited. Goods vehicles whose total weight exceeds 3.5 tons are prohibited. | 11805 |  |  |  |  |
| What is true concerning seat belts in a car?. The seat belt must be used when reversing. The seat belt must be used when driving in a parking area. Taxi drivers do not need to wear seat belts. The driver is obliged to ensure that passengers less than 15 years old use seat belts or special safety devices for children. |
| Which traffic sign shows the maximum permitted bogie load?. A. B. C. D. | 10652 |  |  |  |
|  |  |  |  |  |  |  | What are the consequences of incorrect alignment of the front wheels? |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It is not possible to steer. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | The servo stops working. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It is not possible to brake. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | The tyres become abnormally worn. |  |  |  |  |  |  |  |  |
| Which factors affect kinetic energy?. A car's weight and speed. A car's speed and the condition of the road. A car's weight and the condition of the road. |
| Which tyre indicates faulty front end wheel alignment?. Tyre A. Tyre B. Tyre C. Tyre D. | 49 |  |

| What is true about the car battery?. It should always contain carburettor spirit. During the winter it should contain antifreeze. It contains corrosive acid. | 10501 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| You can feel vibrations in the steering wheel when you drive at a certain speed. What is the most common reason for this problem?. Unbalanced front wheels. The front end needs lubricating. Tyre pressure in the front tyres too high. | 10506 |  |  |  |  |  |  |
| Which statement is correct?. Kinetic energy has no effect during collisions. Kinetic energy has the biggest effect when driving in curves. Standing on a chair and dropping straight down to the floor corresponds to the kinetic energy at 15-20 km/h. |
| Which traffic sign indicates a limited axle load?. Traffic sign A. Traffic sign B. | 8617 |  |  |  |  |  |
| Which vehicles must be equipped with these reflector signs?. Trailers with a total weight exceeding 3.5 tons. All trailers. All heavy goods vehicles. Heavy goods vehicles with a total weight exceeding 3.5 tons. | 11673 |  |  |  |  |  |
|  |  |  |  |  |  | In a certain situation your reaction time is 1 second. How long is the thinking distance if you are driving at 100 km/h? |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Approx. 10 m |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Approx. 20 m |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Approx. 30 m |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Approx. 40 m |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Approx. 50 m |  |  |  |  |  |  |  |  |
| Which warning lamp goes on if the oil pressure is too low?. Lamp no. 3. Lamp no. 6. Lamp no. 8. Lamp no. 9. | 10511 |  |  |  |  |
| In certain batteries you should check the liquid level. Which liquid should you top up the battery with when necessary?. Distilled water.. Salt-solution.. Sulphuric acid.. Methylated spirit.. |
| Which fluid do you mix with water in order to prevent ice from forming in the cooling system?. A. B. C. D. | 10593 |  |  |  |
| You rent a trailer in winter conditions. What is true?. A category BE driving licence is required to tow a rented trailer. If I load the trailer so that it is back-heavy, there is a risk that the car's dipped lights will be dazzling. If the car has studded tyres, the trailer must also have them. If the car has winter tyres without studs, the trailer must also have them. | 10512 |  |  |  |
| You are going to connect the jump leads from an auxiliary battery to your own car to get it started. What is the correct thing to do first?. Connect the auxiliary battery's positive terminal to your battery's minus terminal.. Connect the auxiliary battery's positive terminal to your battery's positive terminal.. Connect the auxiliary battery's plus terminal to part of the bodywork on your own vehicle.. Connect the auxiliary battery's minus terminal to your own battery's plus terminal.. |
| Which sign means that you must be well aware of how wide the vehicle you are driving is?. Traffic sign A.. Traffic sign B.. Traffic sign C.. Traffic sign D.. | 2755 |  |
| How far does the car travel in one second?. At 30 km/h the car travels approx. 8 metres.. At 50 km/h the car travels approx. 22 metres.. At 70 km/h the car travels approx. 32 metres.. At 80 km/h the car travels approx. 34 metres.. |

| Which of the following statements about seat belts is true?. A seat belt combined with a thick jacket reduces the strain on the body in the event of a collision. A seat belt should lie close to the body and be as tight as possible. |
| --- |
| Which lamp lights up when it is time to refuel?. Lamp no. 4. Lamp no. 5. Lamp no. 6. Lamp no. 8. | 1288 |
| Which combination is correct?. Glycol - the radiator.. Carburettor spirit - the battery.. Distilled water - the radiator.. |
| You reduce speed from 90 km/h to 30 km/h. How does this affect the kinetic energy?. It remains unchanged. It becomes three times less. It becomes six times less. It becomes nine times less. |

| Which of the following statements about a temporary spare wheel is true?. It must not be used if the other tyres are studded. It may only be used on dry roads. It wears down faster. A temporary spare wheel may not be used at a higher speed than that printed on the tyre. | 12439 |  |
| --- | --- | --- |
| You are driving at 70 km/h. How far does your vehicle travel in one second?. Approx. 10 m. Approx. 20 m. Approx. 30 m. Approx. 40 m. |
| During what period are you normally allowed to use studded tyres?. 1 October - 31 March. 1 October - 15 April. 1 November - 1 April. 1 November - first Sunday after Easter. | 12773 |

| When must the height of a viaduct be marked out?. Always. When the height available is higher than 4.5 metres. When the height available is lower than 4.5 metres. | 12324 |  |  |
| --- | --- | --- | --- |
|  |  |  | Your car is fitted with studded tyres and you have a puncture. What rule applies? |  |  |  |  |
|  |  |  | As a temporary measure, I can put on any type of tyre |  |  |  |  |
|  |  |  | I must put on an emergency wheel |  |  |  |  |
|  |  |  | I must put on another studded tyre |  |  |  |  |
|  |  |  | What is the minimum height a person must be in order to sit in a front passenger seat fitted with an airbag? |  |  |  |  |
|  |  |  | At least 100 cm |  |  |  |  |
|  |  |  | At least 120 cm |  |  |  |  |
|  |  |  | At least 140 cm |  |  |  |  |
|  |  |  | At least 160 cm |  |  |  |  |
| What does this combination of traffic signs mean?. Minimum permitted distance to the vehicle in front. Vehicles with a width of less than 2.6 meter must have an exemption to drive on the road.. Limited width of vehicle. Limited width of lane. | 2827 |  |

|  |  | Why should the tyres with the best grip be fitted at the rear? |  |
| --- | --- | --- | --- |
|  |  | They are under most strain if I brake hard |  |
|  |  | It shortens the braking distance |  |
|  |  | They are under most strain because of the weight of load in the baggage compartment |  |
|  |  | The car's road-holding characteristics will be better |  |
|  |  | When is there the greatest risk of carbon monoxide poisoning? |  |
|  |  | If I run the car warm for more than one minute. |  |
|  |  | If I have the side window open. |  |
|  |  | If I drive with the wrong fuel. |  |
|  |  | If I reverse into a snowdrift so that the exhaust pipe becomes blocked. |  |
|  |  | What influences the reaction time? |  |
|  |  | The condition of the driver |  |
|  |  | The condition of the vehicle |  |
|  |  | The road conditions |  |
|  |  | The speed |  |
|  |  | You skid as a result of too powerful an acceleration in a curve. What is true about the handling? |  |
|  |  | Front-wheel driven cars skid with the rear of the car |  |
|  |  | Front-wheel driven cars become oversteered |  |
|  |  | Rear-wheel driven cars are experienced as understeered |  |
|  |  | Front-wheel driven cars are experienced as understeered |  |

| What is true about cars with ABS brakes?. The car can be steered during hard braking. The brakes cannot freeze solid. The brakes are maintenance-free. | 10504 |  |  |  |
| --- | --- | --- | --- | --- |
| Where is there information on which type of oil should be used for the engine?. The motor vehicle inspection report.. The instruction manual.. The registration certificate.. |
| Which statement about baby seats in cars is true?. It must be fitted so it rests against the vehicle's dashboard. It must always be fitted in the rear seat. It is the seat belt that restrains the baby seat in a collision. | 12271 |  |  |
| Which statement is correct?. The use of seat belts implies risks in the case of a side collision. The use of seat belts reduces the risks if I have the three-point belt lightly stretched high up on my neck. The use of seat belts increases the risk of injury or death if I turn my car over. The use of seat belts reduces the risk of injury or death in traffic by 40-60 %. |
| Why should the brake fluid be changed every few years?. It becomes viscous. It absorbs water. It absorbs dirt and dust. |
| Which traffic signs are you allowed to pass when driving a light goods vehicle with a coupled trailer? The trailer has an unarticulated towbar.. Traffic signs B and C. Traffic signs C and D. Traffic signs B and D. Traffic signs A and C. Traffic signs A and B. | 11796 |  |
| You are going to jump-start your car by using electricity from another car's battery. Which of the following is correct?. The car that I am taking electricity from must not have its engine on. The minus terminals must be linked to the plus terminals. The batteries that are linked up must have the same voltage. |
| Which lamp lights up if the airbag system is not working?. Lamp no. 1. Lamp no. 2. Lamp no. 9. Lamp no. 10. | 1289 |

| You are driving at 70 km/h. How far does your vehicle travel in one second?. Approx. 10 metres.. Approx. 20 metres.. Approx. 25 metres.. Approx. 35 metres.. |
| --- |
| Lamp no. 6 lights up when you are driving. What should you do?. Press down the clutch and rev up the engine a few times. Continue driving a few kilometres to see if the warning lamp goes off. Stop the engine immediately. | 10510 |  |  |  |  |  |
| What does the concept of thinking distance mean?. The distance the car travels during my reaction time.. The distance the car travels during my reaction time and braking.. The distance the car travels during braking.. |
|  |  |  |  |  |  |  | You are driving a car equipped with airbags for both front seats. Which alternative is correct? |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | It is dangerous to wear glasses if sitting in a seat with an airbag |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | An airbag is triggered in all collisions |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | An airbag provides best protection if I do not use the seatbelt |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | A child safety seat must not be mounted in the front seat |  |  |  |  |  |  |  |  |
| How should a child aged two years most safety be placed in a car?. The child should sit on a booster cushion and use the seat belt. The child should sit in a child safety seat mounted either in the back seat or front seat according to the manufacturer's instructions. The child should sit in a protective cot with seat belt. | 229 |  |  |  |
| Which vehicle or combination of vehicles may pass this sign?. A car with a trailer. A heavy goods vehicle without a trailer. A heavy goods vehicle with a coupled trailer(semitrailer). | 11804 |  |  |  |
| When driving, you notice that the temperature gauge has suddenly gone up to the red area. What is the first thing you should do?. Drive to the nearest garage. Stop and check that there is enough coolant in the cooling system. Stop and check the oil level in the engine. | 10571 |  |  |  |
|  |  |  |  |  |  |  |  | Which statement is correct? |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | An airbag is always turned on and off with a switch on the dashboard |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | If the vehicle has an airbag, you do not need to use a seatbelt |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | A child car seat must not be used in the front passenger seat if there is an airbag in operation |  |  |  |  |  |  |  |
| What is the lowest available height under this bridge?. 3.5 m. 4.5 m. 5.5 m. | 11677 |  |  |
| Which equipment must a car have according to the regulations?. Jump leads.. Fog lights.. Outer rear mirrors.. |
|  |  |  |  |  |  |  |  |  | How tall must a child be in order to use the ordinary seatbelt in a car? |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 115 cm |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 125 cm |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 135 cm |  |  |  |  |  |  |

LEVEL C TEST 3 CODE 13

| What is the lowest available height under this bridge?. 3.5 m. 4.5 m. 5.5 m. | 11677 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Which traffic sign or signs provide information on the maximum gross weight of a vehicle or vehicle train that is allowed to pass?. Only sign A. Only sign B. Traffic sign A and C. Traffic sign B and D. Traffic sign A and B. | 8859 |  |  |  |  |
| You increase speed from 30 km/h to 60 km/h. How does this affect the kinetic energy?. It remains unchanged. It becomes twice as large. It becomes three times as large. It becomes four times as large. |
|  |  |  | Which statement is correct? |  |  |  |  |  |  |
|  |  |  | An airbag is always turned on and off with a switch on the dashboard |  |  |  |  |  |  |
|  |  |  | If the vehicle has an airbag, you do not need to use a seatbelt |  |  |  |  |  |  |
|  |  |  | A child car seat must not be used in the front passenger seat if there is an airbag in operation |  |  |  |  |  |  |
| Why should the brake fluid be changed every few years?. It becomes viscous. It absorbs water. It absorbs dirt and dust. |
| Which traffic sign indicates a limited axle load?. Traffic sign A. Traffic sign B. | 8617 |  |  |  |
| What is true?. If there is too little air in the left-hand front tyre the car pulls to the right. If there is too little air in the right-hand front tyre the car pulls to the right. |
| You are driving a vehicle whose height exceeds 2.5 m. What does this traffic sign combination mean?. Driving straight ahead is permitted but after 900 m there will be limited available height. I must turn right. Driving straight ahead is prohibited. | 11991 |  |

- You notice that your car is pulling to one side. What could be the reason? The power steering is not working High air pressure in both front tyres Low air pressure in a front tyre

| There is not enough fluid in the car battery. What should you top up with?. Distilled water. Carburettor spirit. Antifreeze. | 10509 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Which statement is correct?. I must use the seat belt when I am driving in a parking area. Adult passengers only need to use the seat belt in built-up areas. I must always use the seat belt when I am reversing. Adult passengers are responsible themselves for using the seat belt. |
| Which statement is true regarding studded tyres?. They may always be used if there is a risk of wintry road conditions. They may be used all the year round, no matter the road conditions. They increase the risk of aquaplaning. | 10585 |  |  |  |  |  |  |  |  |  |  |
| What does this traffic sign mean?. Goods vehicles whose maximum load exceeds 3.5 tons are prohibited. Goods vehicles whose gross weight exceeds 3.5 tons are prohibited. Goods vehicles whose total weight exceeds 3.5 tons are prohibited. Goods vehicles whose kerb weight exceeds 3.5 tons are prohibited. | 11805 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | The thinking distance increases in relation to the speed. What is correct? |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Twice the speed gives a thinking distance twice as long. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Twice the speed gives a thinking distance three times as long. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Twice the speed gives a thinking distance four times as long. |  |  |  |  |  |  |  |  |  |  |  |  |
| When must the height of a viaduct be marked out?. Always. When the height available is lower than 4.5 metres. When the height available is higher than 4.5 metres. | 12324 |  |  |  |  |  |  |  |  |  |
| You want to start your car with the help of another car's battery. Which statement is correct?. The batteries must be of different voltages (number of volts). The engine of the car providing starting help must be turned off. If the batteries are connected incorrectly, there may be a short circuit. | 10613 |  |  |  |  |  |  |  |  |  |
| What is true about cars with ABS brakes?. The car can be steered during hard braking. The brakes are maintenance-free. The brakes cannot freeze solid. | 10504 |  |  |  |  |  |  |  |  |
| Which lamp lights up if the airbag system is not working?. Lamp no. 1. Lamp no. 2. Lamp no. 9. Lamp no. 10. | 1289 |  |  |  |  |  |  |
| Which statement is true regarding car batteries?. They contain a corrosive acid. They work at full capacity even at very low temperatures. They must contain glycol in the winter so as not to freeze.. | 10576 |  |  |  |  |  |
| Which traffic signs are you allowed to pass when driving a light goods vehicle with a coupled trailer? The trailer has an unarticulated towbar.. Traffic signs B and C. Traffic signs B and D. Traffic signs A and B. Traffic signs C and D. Traffic signs A and C. | 11796 |  |  |  |  |  |
| How can you check that the ABS brake is working?. A warning light flashes when I brake hard. The brake pedal throbs and pulses when I brake hard. The front tyres smoke when I brake hard. |
| Which vehicle or combination of vehicles may pass this sign?. A car with a trailer. A heavy goods vehicle with a coupled trailer(semitrailer). A heavy goods vehicle without a trailer. | 11804 |  |  |  |  |
| Which tyre indicates faulty front end wheel alignment?. Tyre A. Tyre B. Tyre C. Tyre D. | 49 |  |  |  |  |
| Which fluid do you mix with water in order to prevent ice from forming in the cooling system?. A. B. C. D. | 10593 |  |  |  |  |
| Which sign means that you must be well aware of how wide the vehicle you are driving is?. Traffic sign A.. Traffic sign B.. Traffic sign C.. Traffic sign D.. | 2755 |  |  |  |  |
| Which equipment must a car have according to the regulations?. Two red reflectors at the rear.. Tachometer.. Jack.. |
| Which lamp lights up when the brake fluid level is too low?. Lamp no. 1. Lamp no. 6. Lamp no. 8. Lamp no. 9. | 1285 |  |  |  |
| What does the concept of stopping distance mean?. The distance the car travels during braking.. The distance the car travels during my reaction time.. The total length of the thinking distance and braking distance.. |
| Which vehicles must be equipped with these reflector signs?. Heavy goods vehicles with a total weight exceeding 3.5 tons. All heavy goods vehicles. Trailers with a total weight exceeding 3.5 tons. All trailers. | 11674 |  |  |
| Which of these accessories should there be in a car?. Spare tyre. Jump leads. Towline. Triangular warning sign. |
| What is the minimum height a person must be in order to sit in a front passenger seat fitted with an airbag?. At least 100 cm. At least 120 cm. At least 140 cm. At least 160 cm. |
| Which traffic sign shows the maximum permitted bogie load?. A. B. C. D. | 10652 |  |
| When can children travel in a car using only the car's seatbelt?. When they are at least 135 cm long. When they are 7 years old. When they weigh over 50 kg. |
| You reduce speed from 100 km/h to 50 km/h. How does this affect the braking distance?. It remains unchanged. It becomes half as long. It reduces to a third. It reduces to a fourth. |
| What is true concerning the engine heater?. It increases the fuel consumption since a hot engine uses more fuel than a cold engine. It makes the work of the catalytic converter more difficult. It should only be used when the temperature outside is below minus. It reduces the fuel consumption and therefore the emission of exhaust fumes. |
| Your car has no child safety-seat. Is there ever an occasion when you may transport a child who is 4 years old and about 100 cm long?. Yes, no matter the distance, as a one-off I may transport a child in the backseat. No, children must always travel in a special child safety-seat. Yes, but only a short distance and only if child sits in the back seat. |
| You are driving a car equipped with airbags for both front seats. Which alternative is correct?. It is dangerous to wear glasses if sitting in a seat with an airbag. An airbag is triggered in all collisions. A child safety seat must not be mounted in the front seat. An airbag provides best protection if I do not use the seatbelt. |

| Lamp no. 6 lights up when you are driving. What should you do?. Press down the clutch and rev up the engine a few times. Stop the engine immediately. Continue driving a few kilometres to see if the warning lamp goes off. | 10510 |  |  |
| --- | --- | --- | --- |
| Which statement is true regarding the steering system?. If the steering wheel vibrates, it is because the fluid level in the power steering is too low. It is not possible to steer a car with power steering if the engine is turned off. If the steering wheel catches when I turn, then air or dirt has probably got into the power steering system. |
|  |  |  | What is true concerning seat belts in a car? |  |  |  |
|  |  |  | The seat belt must be used when reversing |  |  |  |
|  |  |  | The seat belt must be used when driving in a parking area |  |  |  |
|  |  |  | The driver is obliged to ensure that passengers less than 15 years old use seat belts or special safety devices for children |  |  |  |
|  |  |  | Taxi drivers do not need to wear seat belts |  |  |  |
| Which lamp lights up when it is time to refuel?. Lamp no. 4. Lamp no. 5. Lamp no. 6. Lamp no. 8. | 1288 |  |
| Which of the following statements about a car's warning lights is true?. If the charging system lamp comes on while I am driving, I must stop immediately and check the fluid level of the battery. If the red oil pressure lamp comes on while I am driving, I must stop immediately and turn off the engine. If the temperature gauge suddenly rises to boiling point, I must stop immediately and check the engine's oil level. When is there the greatest risk of carbon monoxide poisoning?. If I run the car warm for more than one minute.. If I have the side window open.. If I reverse into a snowdrift so that the exhaust pipe becomes blocked.. If I drive with the wrong fuel.. Do you have to wear a seat belt when travelling in a taxi?. No. Yes.; 1365. What does this combination of traffic signs mean?. Minimum permitted distance to the vehicle in front. Vehicles with a width of less than 2.6 meter must have an exemption to drive on the road.. Limited width of lane. Limited width of vehicle.; 2827. | 11925 |  |

- You are driving at 70 km/h. How far does your vehicle travel in one second? Approx. 10 m Approx. 20 m Approx. 30 m Approx. 40 m

|  |  |  | In certain batteries you should check the liquid level. Which liquid should you top up the battery with when necessary? |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Distilled water. |  |  |  |
|  |  |  | Salt-solution. |  |  |  |
|  |  |  | Methylated spirit. |  |  |  |
|  |  |  | Sulphuric acid. |  |  |  |
| Which lamp lights up when the battery is not charging?. Lamp no. 6. Lamp no. 7. Lamp no. 9. Lamp no. 10. | 1292 |  |
| You can feel vibrations in the steering wheel when you drive at a certain speed. What is the most common reason for this problem?. Unbalanced front wheels. Tyre pressure in the front tyres too high. The front end needs lubricating. | 10506 |  |

- Which statement is correct? The hip strap on the seat belt should be placed over the soft part of the stomach A hard stretched belt often causes injuries during a collision The force of a collision at 50 km/h is comparable with falling from the third floor As the driver I am responsible for passengers over 15 years of age using seat belts

| By how much is motive energy reduced and therefore the braking distance if you reduce your speed from 100 kph to 50 kph?. By half. To a third. To a quarter. |
| --- |
|  |  |  |  |  |  |  |  |  |  | Are you allowed to drive a car whose foot brake is not working? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | No, I must call in a breakdown truck |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Yes, if you use the hazard warning lights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Yes, the shortest possible route to a garage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Which statement is correct? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Kinetic energy has no effect during collisions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Standing on a chair and dropping straight down to the floor corresponds to the kinetic energy at 15-20 km/h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Kinetic energy has the biggest effect when driving in curves |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | How far does the car travel in one second? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | At 30 km/h the car travels approx. 8 metres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | At 50 km/h the car travels approx. 22 metres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | At 70 km/h the car travels approx. 32 metres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | At 80 km/h the car travels approx. 34 metres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Which vehicles must be equipped with these reflector signs?. Trailers with a total weight exceeding 3.5 tons. All trailers. Heavy goods vehicles with a total weight exceeding 3.5 tons. All heavy goods vehicles. | 11673 |  |  |  |  |  |  |  |  |  |  |
| What is meant by stopping distance?. It is the distance my vehicle travels from when I see danger to when the vehicle stands still. It is the distance my vehicle travels from when I see danger to when I start braking. It is the distance my vehicle travels from when I start braking until the vehicle stands still. |
| You increase speed from 30 km/h to 90 km/h. How does this affect the braking distance?. It remains unchanged. It becomes three times as long. It becomes six times as long. It becomes nine times as long. |
| You rent a trailer in winter conditions. What is true?. A category BE driving licence is required to tow a rented trailer. If I load the trailer so that it is back-heavy, there is a risk that the car's dipped lights will be dazzling. If the car has winter tyres without studs, the trailer must also have them. If the car has studded tyres, the trailer must also have them. | 10512 |  |  |  |  |  |  |  |  |
| You are driving at 70 km/h. How far does your vehicle travel in one second?. Approx. 10 metres.. Approx. 20 metres.. Approx. 25 metres.. Approx. 35 metres.. |
| Which lamp lights up when the engine temperature is too high?. Lamp no. 3. Lamp no. 6. Lamp no. 7. Lamp no. 8. | 1287 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Which of the following statements about seat belts is true? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | A seat belt combined with a thick jacket reduces the strain on the body in the event of a collision |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | A seat belt should lie close to the body and be as tight as possible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Which warning lamp goes on if the oil pressure is too low?. Lamp no. 3. Lamp no. 6. Lamp no. 8. Lamp no. 9. | 10511 |  |  |  |
| Which traffic sign is applicable to the next crossing?. A. B. C. | 1081 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | You are going to connect the jump leads from an auxiliary battery to your own car to get it started. What is the correct thing to do first? |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Connect the auxiliary battery's positive terminal to your battery's minus terminal. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Connect the auxiliary battery's positive terminal to your battery's positive terminal. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Connect the auxiliary battery's minus terminal to your own battery's plus terminal. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Connect the auxiliary battery's plus terminal to part of the bodywork on your own vehicle. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When driving, you notice that your car pulls to the right. What could the reason be?. There is too little air in the left front tyre. The tyres are worn to different degrees. There is too little air in the right front tyre. |
| Where is there information on which type of oil should be used for the engine?. The motor vehicle inspection report.. The registration certificate.. The instruction manual.. |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Which combination is correct? |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Glycol - the radiator. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Distilled water - the radiator. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Carburettor spirit - the battery. |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Which statement is correct concerning the driver's responsibility for the use of seat belts? |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | The driver is always obliged to use the seat belt even in a parking area |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | The driver of a light lorry does not need to use the seat belt |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | The driver is only responsible to ensure that children less than 15 years of age use a seat belt or special safety device |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | The driver is responsible to ensure that everyone in the car uses the seat belts |  |  |  |  |  |  |  |  |  |  |  |  |
| Which of the following must a car be fitted with?. Spare wheel. Pre-engine heater. Catalytic converter. Cruise control. | 12128 |  |  |  |
| What is applicable when children travel in a car?. There are no rules concerning how children less than 6 years old should be secured. All children must be "secured" in a suitable way for their age. A small child sitting on the knee of an adult can be secured in the same seat belt as the adult. |